聖公會呂明才中學

S.K.H. Lui Ming Choi Secondary School

創科班 Little Makers Class

單元二：動手做 STEAM  Unit 2: Hands-on STEAM

第 4 堂｜結構力學——紙橋承重挑戰

Lesson 4 | Structural Mechanics — Paper Bridge Load Challenge

教師筆記

Teacher Notes

本堂物資表 Materials List（教師版）

請預早一星期準備所需物資；標示「課室現有」之項目請預先清點。

| 品名 Item | 數量（每組） Quantity (per group) | 大約成本 Est. cost | 安全注意 Safety notes |
| --- | --- | --- | --- |
| A4 影印紙 A4 copy paper | 10 張 10 sheets | 約 HK$2 approx. HK$2 | 紙邊或割傷手指，小心使用 Paper edges may cause small cuts; handle with care |
| 透明膠紙 Adhesive tape | 1 卷 1 roll | 約 HK$5 approx. HK$5 | 剪刀使用注意安全 Take care when using scissors |
| 硬幣（$1） Coins ($1) | 50 枚（全班共用） 50 pieces (shared by class) | 向學生借用，無成本 Borrowed from students; no cost | 硬幣細小，課後須點算收回 Small items; count and collect after class |
| 小紙碟 Small paper plates | 每組 2 個 2 per group | 約 HK$1 approx. HK$1 | — — |
| 剪刀 Scissors | 每組 1 把 1 per group | 課室現有 Available in classroom | 刀口鋒利，傳遞時刀柄先行 Blades are sharp; pass handle-first |
| 書桌 Desks | 每組 2 張 2 per group | 課室現有 Available in classroom | — — |
| 間尺 Ruler | 每組 1 把 1 per group | 課室現有 Available in classroom | — — |

本文件包含學生筆記所有題目的參考答案、家課答案、附加思考題、建議教導方向及常見迷思，僅供教師使用，切勿派發給學生。

This document contains reference answers to all student-note questions, homework answers, extra thinking questions, teaching suggestions and common misconceptions. For teachers only — do not distribute to students.

一、學生筆記參考答案

1. Student Notes — Reference Answers

簡單問答題  Quick questions

■ 參考答案  Reference answers

1. 橋不會塌下，因為結構把重量（力）傳到橋墩和地基；可接受答案：三角形／拱形結構穩固、材料強度足夠、設計經過計算等。

A bridge stands because its structure transfers weight (forces) to the piers and foundations; accept: triangular/arch structures are stable, materials are strong enough, the design is calculated.

2. 三角形框架不會變形。因為三角形三條邊的長度固定後，形狀便唯一確定；四邊形邊長固定仍可改變角度，變成平行四邊形。

The triangular frame does not deform. With three side lengths fixed, a triangle's shape is uniquely determined; a quadrilateral can still change its angles and become a parallelogram.

3. 硬幣放在中央小紙碟上，可使重量平均分布在橋面中央，避免硬幣邊緣直接壓穿紙面造成局部破損；放在中央亦是橋樑受力最大的位置，最能考驗結構。

Coins on the central plate spread the weight evenly at the middle of the deck and prevent coin edges from punching through the paper; the centre is also where the bridge bears the most load, giving the fairest test.

思考題  Thinking questions

■ 參考答案  Reference answers

1. 應選「改變形狀」。因為承重能力主要由結構形狀決定（三角形、拱形、摺疊），單純加紙增加重量之餘，效益遞減；形狀改得好，用更少紙可承更重。

Choose "change the shape". Load capacity is mainly set by structural shape (triangles, arches, folds); simply adding paper adds weight with diminishing returns, while a better shape carries more with less paper.

2. 因為摺疊令紙在厚度方向變得更「高」，更難被壓彎（抗彎能力大增）；單純疊厚五層，每層之間沒有結構連結，仍容易整體彎曲。關鍵是形狀，不是厚度。

Folding makes the paper "taller" in the thickness direction so it resists bending far better; five simply-stacked layers have no structural connection between them and still bend as a whole. The key is shape, not thickness.

課堂小測驗參考答案  Class quiz — reference answers

■ 參考答案  Reference answers

1. C（三角形）

C (triangle)

2. B（紙在厚度方向變得更難被壓彎）

B (the paper becomes harder to bend in the thickness direction)

3. A（橋面中央的小紙碟上）

A (on the small paper plate at the centre of the deck)

4. 參考答案：橋樑承重靠結構把力傳走；三角形、拱形等形狀能把重量分散到橋墩，單靠加材料只會增加自身重量，效益低。評分：能說出「形狀傳力／分散重量」得滿分。

Reference: a bridge carries load by channelling forces through its structure; shapes like triangles and arches spread weight to the piers, while merely adding material also adds self-weight with low benefit. Full marks for mentioning "shape channels/spreads forces".

5. 乙組較有效率。乙組平均每張紙承起 15 ÷ 6 = 2.5 枚，甲組平均每張紙承起 12 ÷ 10 = 1.2 枚；乙組用較少紙承起較多硬幣。

Group B is more efficient: 15 ÷ 6 = 2.5 coins per sheet versus Group A's 12 ÷ 10 = 1.2 coins per sheet; Group B uses less paper to support more coins.

二、家課參考答案

2. Homework — Reference Answers

■ 參考答案  Reference answers

1. C（三角形桁架）

C (triangular truss)

2. B（把重量沿弧線傳到兩端的橋墩）

B (channels weight along the curve to the supports at both ends)

3. B（摺疊令紙更難被壓彎）

B (folding makes the paper harder to bend)

4. C（重量集中，紙面容易被壓穿）

C (the weight is concentrated and can punch through the paper)

5. B（乙組平均每張紙承起 2.5 枚，高於甲組的 1.2 枚）

B (Group B averages 2.5 coins per sheet, higher than Group A's 1.2)

6. 參考答案：我組採用三角形桁架結構，橋面摺成 W 形；測試中共承起 16 枚硬幣。三角形三邊固定使形狀不變形，W 形摺疊令紙更難彎曲，所以承重能力高。（須包含：結構類型、測試結果、原因解釋。）

Reference: our group used a triangular truss with a W-folded deck and supported 16 coins. The triangle's fixed sides prevent deformation and the W-fold resists bending, hence the high load capacity. (Must include: structure type, test result, explanation.)

7. 參考答案示例：下次會在橋面兩端加三角形支撐，並把膠紙接口改在受力較小的位置，因為今次倒塌是接口先鬆開。（評分：有具體改良＋有原因。）

Sample: next time we would add triangular supports at both ends and move taped joints to lower-stress positions, because this time the joints loosened first. (Mark: concrete improvement + reason.)

8. 反思短文評分：內容須包括設計意念、測試結果、「再做一次如何改良」三部分；不少於 80 字；語句通順。每缺一部分扣約三分之一分數。

Reflection marking: must cover design idea, test results, and "how to improve next time"; at least 80 Chinese words; coherent writing. Deduct about one-third for each missing part.

三、附加思考題

3. Extra Thinking Questions

■ 參考答案  Reference answers

1. 現實中的行人天橋多用鋼材而不用紙。除了材料強度，工程師還會考慮甚麼因素？（提示：風、行人走動、耐用年期）參考答案：風力造成的搖晃、行人走動引起的震動、長期日曬雨淋的耐用性、維修成本等。

Real footbridges use steel, not paper. Besides material strength, what else do engineers consider? (Hint: wind, pedestrians, lifespan) Reference: swaying caused by wind, vibration from pedestrians, durability under sun and rain, maintenance costs, etc.

2. 為甚麼測試規則要求「兩端各留 5 厘米承托」，而不准橋身整條貼在書桌上？參考答案：承托位模擬真實橋墩；若整條橋貼在桌上，桌面會分擔重量，測不出結構本身的承重能力。

Why do the rules require "5 cm of support at each end" instead of laying the whole bridge on the desks? Reference: the supports model real piers; if the whole bridge rested on the desks, the desks would share the weight and the structure's own capacity could not be tested.

3. 如果改用五元硬幣（較重）測試，承重數目會怎樣變？為甚麼？參考答案：數目會減少，因為每枚硬幣更重，總重量上限不變，能承受的枚數自然較少；這說明應比較「總重量」而非只看枚數。

If $5 coins (heavier) were used instead, how would the count change? Why? Reference: the count would drop because each coin is heavier; the total weight limit is unchanged, so fewer coins can be supported — showing that total weight, not just count, should be compared.

四、建議教導方向

4. Suggested Teaching Approach

1. 引入（0–10 分鐘）：展示青馬大橋相片，先問「橋為何不會塌下」，收集學生答案後再派材料；緊扣規則：跨度 30 厘米、兩端各 5 厘米承托。

Introduction (0–10 min): show the Tsing Ma Bridge photo, ask "Why doesn't a bridge collapse?", collect answers, then distribute materials; stress the rules: 30 cm span, 5 cm support at each end.

2. 設計（10–25 分鐘）：先簡介三角形、拱形、樑式三種結構，巡視時多提問少代答，例如「你覺得重量會往哪裡走？」引導學生思考傳力路徑。

Design (10–25 min): brief the three structures, then visit groups asking guiding questions rather than giving answers, e.g. "Where do you think the weight will go?", to prompt thinking about force paths.

3. 搭建與測試（25–50 分鐘）：提醒硬幣放在中央小紙碟上；要求學生即時填寫數據記錄表，不要靠記憶後補。

Build and test (25–50 min): remind students to place coins on the central plate; require the data table to be filled in during testing, not from memory afterwards.

4. 總結（50–60 分鐘）：各組匯報後，引導全班比較「用紙量 vs 承重數」，點出形狀比材料更重要的結論；預告下堂橡筋動力車。

Conclusion (50–60 min): after group reports, guide the class to compare "paper used vs coins supported" and draw the conclusion that shape matters more than material; preview the next lesson on rubber-band cars.

五、常見迷思

5. Common Misconceptions

1. 「用紙愈多愈穩固」→ 引導比較各組「每張紙平均承重數」，讓學生從數據發現結構形狀才是關鍵。

"More paper means more strength" → guide students to compare "coins per sheet" across groups so the data shows that shape is the key.

2. 「紙愈厚愈好」→ 引導比較「疊厚五層」與「摺成波浪形」的分別，說明摺疊改變形狀、增加抗彎能力，單純加厚效益低。

"Thicker paper is better" → prompt comparison of "five stacked layers" versus "wavy fold", showing that folding changes shape and resists bending while mere thickening has low benefit.

3. 「硬幣放哪裡都一樣」→ 讓學生試把硬幣放在橋端，觀察橋身一側下塌，理解重量位置影響受力。

"It doesn't matter where the coins go" → let students try placing coins near one end and watch that side collapse, to understand that load position affects forces.